ICLOROX

The Clorox Company

1221 Broadway Oakland, California 94612 Phone: 510-271-7000

Material Safety Data Sheet

| Product: lemon fresh pine-sol® cleaner & antibacterial spray |  |  |
| --- | --- | --- |
| Description: clear yellow liquid with citrus odor |  |  |
| Other Designations Distributor Emergency Telephone Nos. |  |  |
| Clorox Sales Company For Medical Emergencies, call EPA Registration Number 581 3-55 1221 Broadway Rocky Mountain Poison Center: 1-800-446-1014 Oakland, CA 94612 For Transportation Emergencies, call Chemtrec: 1-800-424-9300 |  |  |
| II Health Hazard Data | Ill Hazardous Ingredients |  |
| Eye irritant. Direct or prolonged skin contact may result in irritation. Avoid contact with food. Overexposure to this product may cause the Worker Exposure Limit of 2-butoxyethanol to be exceeded. 2-ButoxyethanoT has been shown to cause red blood cell hemolysis in laboratory animals and secondary injury to the kidney and liver. However, humans appear to be resistant to this effect. No medical conditions are known to be aggravated by exposure to this product. FIRST AID: | Ingredient Concentration Worker Exposure Limit |  |
|  | Alkyl(C,240%;C,450%; 0.3% Not established. C,610%) dimethyl benzyl |  |
|  | ammonium chloride CAS # 68424-85-1 2-Butoxyethanol 1 - 5% 25 ppm - skin TLV-TWA" CAS #111 -76-2 50 ppm - skin PEL" Note that 2-butoxyethanol can be absorbed through the skin. This may contribute to overall exposure. 'TLV-TWA = ACGIH Threshold Limit Value - Time Weighted Average |  |
| EYE CONTACT: Flush eyes with plenty of water for 15 minutes. If |  |  |
| irritation occurs, call a physician. SKIN CONTACT: Wash skin with water. If irritation persists, call a |  |  |
| physician. INGESTION: Drink a glassful of water. Call a physician. | None of the materials in this product are on the IARC, OSHA, or NTP carcinogen lists. |  |
| IHALATION: Remove to fresh air. If respiratory irritation develops or |  |  |
| ureathlng becomes difficult, call a physician. |  |  |
| IV Special Protection and Precautions | V Transportation and Regulatory Data |  |
| Hygienic Practices: Wash hands after direct contact. | DOT/IATA/IMO: Not restricted. |  |
| Engineering Controls: Use local exhaust to minimize exposure to product | TSCA Status: All components of this product are on the TSCA inventory. |  |
| mist. Personal Protective Equipment: Wear safety glasses. Wear rubber or | EPA - SARA Title III/CERCLA: Packaged product \s not reportable under |  |
|  | Sections 31 1/312. This product contains no chemicals which are regulated under Section 313 and contains glycol ethers (<5%) and sodium hydroxide (CAS #1310-73-2, <0.5%), which are regulated under c;srtirtn ^n^'PFOP1 A |  |
| neoprene gloves if there is the potential for repeated or prolonged skin contact. |  |  |
| VI Spill Procedures/Waste Disposal | VII Reactivity Data |  |
| Spill Procedures: Absorb and containerize. Wash residual down to | Stable under normal use and storage conditions. |  |
| sanitary sewer. Contact the sanitary treatment facility in advance to assure ability to process washed-down material. Waste Disposal: Dispose of in accordance with all applicable federal. |  |  |
| state, and local regulations. |  |  |
| VIII Fire and Explosion Data | IX Physical Data |  |
| Flash Point: >200°F (closed cup) | pH . -12.5 |  |
| Fire Extinguishing Agents: Dry chemical, carbon dioxide (CO?), foam or | Specific gravity -1 .0 |  |
|  | Solubility in water Soluble |  |
| water spray. |  |  |

C1983. 1991 THE CLOROX COMPANY DATA SUPPLIED IS FOR USE ONLY IN CONNECTION WITH OCCUPATIONAL SAFETY AND HEALTH

DATE PREPARED    1 0/97